## One-day Dictyostelium myosin preparation (One Day Protocol)

References:	Uyeda and Spudich, Science 262: 1867-1870

Ruppel et al., JBC. 269: 18773-18780

This protocol involves rounds of myosin assembly and disassembly. Myosins capable of forming filaments, therefore, are crucial for a high yield. Cells growing at log-phase are essential for obtaining active myosin. All procedures after step 1 are performed at 0-50C.

### Buffers

| Lysis buffer: | Wash buffer: |
| --- | --- |
| 25 mM HEPES pH 7.4 | 10 mM HEPES pH 7.4 |
| 55 mM NaCl | 150 mM NaCl |
| 2 mM EDTA | 1 mM EDTA |
| 1 mM DTT | 1 mM DTT |

| Extraction buffer: | Precipitation buffer: |
| --- | --- |
| 10 mM HEPES pH 7.4 | 10 mM HEPES pH 7.4 |
| 200 mM NaCl | 10 mM MgCL2 |
| 3 mM MgCl2 | 1 mM EDTA |
| 2 mM ATP | 1 mM DTT |
| 1 mM DTT |  |

| Myosin storage buffer: | Kinase buffer: |
| --- | --- |
| 10 mM HEPES pH 7.4 | 10 mM HEPES pH 7.4 |
| 200 mM NaCl | 3 mM MgCl2 |
| 1 mM MgCl2 | 2 mM ATP |
| 0.5 mM ATP | 1 mM DTT |
| 1 mM DTT |  |

Dialysis buffer:

10 mM Pipes pH 6.5

50 mM NaCl

10 mM MgCl2

1 mM DTT

### Protocol

1.	Harvest cells.

2. 	Wash in 10 mM Tris-HCl, 1 mM EDTA. Weigh cell pellet.

3.	Resuspend the cell pellet in 4 vol/g cells of lysis buffer.

4.	Add 4 vol/g cells of lysis buffer containing 1% Triton X-100 and a cocktail of proteinase inhibitors (PMSF, TPCK, pepstatin, leupeptin at concentrations suggested in this Lab Manual) while gentle swirling. Keep on ice for 5 min. Ckeck the lysis under scope.

5. 	Spin 20k x 30 min in SS34 rotor, discard the supe by hand pipetting.

6. 	Resuspend the pellet in 1 vol/g cells of wash buffer. Dilute 3 fold with a buffer containing 10 mM HEPES, pH 7.4, 1 mM EDTA, 1 mM DTT.

7.	Spin 20k x 30 min in SS34, Discard the supe. Be sure to remove any lipids floating in the supe.

8. 	Resuspend the pellet in 4 vol/g cells of extraction buffer.

9.	Spin 52k x 30 min in Ti70 rotor.  Remove lipids (if present) in the upper portion of the supe.

10.	Dilute the supe 4 fold with precipitation buffer containing 20 g/ml RNAase A (boiled before use), and incubate 20 min on ice.

11.	Spin 42k x 60 min in Ti45 rotor.

12.	Homogenize the pellet in 0.3-0.5 vol/g cells of extraction buffer.

13.	Spin 40k x 8 min in Ti70.1 rotor.

14.	Dilute the supe 4 fold with precipitation buffer. Incubate on ice for 20 min.

15.   	Sin 52k x 30 min in Ti70.

16. 	Dissolve the pellet in 10-20 ul/g cells of myosin solubilizing buffer. Clarify by spinning at 15k x 5 min in table top centrifuge if necessary.

*To achieve improved yield, Steps 14 and 15 can be substituted with the following: dialyze the step 13 supe O/N against dialysis buffer. White precipitate forms. Spin 20k x 30 min in SS34.

## Phosporylation of myosin by MLCK

Phosphorylation by the Dictyostelium MLCK is sometimes necessary to obtain active myosin purified by this protocol.

Add 9 vol kinase buffer and appropriate amount of recombinant MLCK (the bacterial expression strain obtained from J. Spudich). Incubate at room temperature for 20 min.

Add 10 mM MgCl2 and incubate for 20 min on ice. Spin 75k x 15 min in TLA100.3 rotor.

Dissolve the pellet in myosin storage buffer.

* To get better recovery, the starting myosin concentration should be above 2 mg/ml.